Appendix C1. The bandwidth h for kernel density estimate (KDE) plot

The first type of option for bandwidth of KDE in LDAC is to provide a fixed bandwidth, such as ‘Silverman331’ (Silverman, 1998, p. 47-48),

h=0.9An-1/5 and A=MinsDe,IQR/1.34					(C1.1)

where sDe is the standard deviation of observed equivalent dose (De) values and IQR is the interquartile range, the difference between upper and lower quartiles. Larger bandwidth h yields less graphical peaks, potentially obscuring data subpopulations. In contrast, a smaller h may yield insights into multiple populations, but it might display more essentially ‘noise’ (Silverman, 1998, p. 7-50, Galbraith and Roberts, 2012; Vermeesch, 2012). Users can also select the ‘User-defined’ in the list to input a user-specified value.

The second type of option is ‘Adaptive’ bandwidth (Botev et al., 2010; Vermeesch, 2018), which replaces the fixed bandwidth h in Eq. (9) of the main text with a local bandwidth hi=hλi. The local parameter λi is calculated by

λi=G/KDE(di),								(C1.2)

lnG=i=1nlnKDE(di)n,								(C1.3)

where di is the observed value for each aliquot or grain i; KDE(di) is the density at point di evaluated by a fixed bandwidth h; n is the number of valid data points. If the ‘Adaptive’ is chosen, the h will be derived from Eq. (C1.1). If the ‘User-adaptive’ is chosen, the constant h is equal to the input value.

The third option is the probability density functions (PDF) when ‘PDF plot’ is chosen, where the bandwidth h in Eq. (9) is replaced by the analytical uncertainties σi. Thus, the PDF for a set of observed values di±σi,

PDFx=1ni=1n1σi2πexp-12x-diσi2,					(C1.4)

The high precision data in PDF will have higher peaks, and low precision data are smoothed out by a broad kernel (Galbraith and Robert, 2012).

Appendix C2. Statistical principle of common age model and finite mixture model (FMM).

- Common age model

The common age model assumes that the true equivalent dose (De) may be the same or very similar for all aliquots or grains, and consistent with a common value with no additional dispersion to consider. This common value μ of a set of observed values di±σi for aliquot or grain i is usually estimated by a standard weighted average method (Taylor, 1997, p173-179; Galbraith, 2005, p. 47-50; Galbraith and Robert, 2012),

μ=i=1nwidii=1nwi									(C2.1)

wi=1σi2.									(C2.2)

The standard error of the maximum likelihood estimate μ could be approximately:

se μ= 1i=1nwi .								(C2.3)

This method also known as ‘inverse variance weighted mean’ (Taylor, 1997, p. 173-179; Galbraith and Roberts, 2012).

This common age model can also be applied to log-transformed data, especially for the data set that are positively skewed. In this case, the di and σi in Eqs. (C2.1-C2.3) are the natural log of observed De value and its relative standard error, respectively.

- Finite Mixture model (FMM)

The FMM assumes that a sample contains several discrete De components, where individual components represent different depositional events (Galbraith and Robert, 2012). The FMM can be employed to identify these individual components from mixture sediments. The statistical principle of the FMM can be referred to Galbraith (2005, chapter 5). This discrete mixture is described by:

j=1kπj=1								(C2.4)

si2=σwi2+σb2								(C2.5)

fji=Ndiμj,si2								(C2.6)

fi=j=1nfji								(C2.7)

where k is the number of components, πj is the proportion of the aliquots or grains that belong to the jth component, thus the sum of πj is 1; μj is the mean of the jth component;di and σwi are the natural log of De and relative standard error (observed values and absolute errors in normal scale); the σb is an additional dispersion, as with in minimum age model (MAM); fji is the contribution of di to the jth components. The unknown parameters πj and μj can be estimated when the log-likelihood L reaches a maximum (Galbraith,2005, p. 90-105), where

L=i=1nln⁡fi.									(C2.8)

The number of k in LDAC for given data sets is evaluated by the Bayes Information Criterion (BIC) method (Galbraith and Roberts, 2012):

BIC=-2Lmax+klnn							(C2.9)

where k is the number of fitted parameters, n is the number of observed values, and the Lmax is the maximum likelihood. The most appropriate component number in Eq. (C2.4) corresponds to the smallest BIC (Galbraith and Roberts, 2012).

References:

Botev, Z.I., Grotowski, J.F., Kroese, D.P., 2010. Kernel density estimation via diffusion. The annals of Statistics 38, 2916-2957.

Galbraith, R.F., 2005. Statistics for fission track analysis. Chapman and Hall/CRC.

Galbraith, R.F., Roberts, R.G., 2012. Statistical aspects of equivalent dose and error calculation and display in OSL dating: an overview and some recommendations. Quat. Geochronol. 11, 1-27.

Silverman, B.W., 1998. Density estimation for statistics and data analysis. Routledge.

Taylor, J., 1997. An Introduction to error analysis: the study of uncertainties in physical measurements. University Science Books, Sausalito, California.

Vermeesch, P., 2012. On the visualisation of detrital age distributions. Chem Geol 312, 190-194.

Vermeesch, P., 2018. IsoplotR: A free and open toolbox for geochronology. Geoscience Frontiers 9, 1479-1493.